24-650 Applied Finite Element Analysis

Assignment 1

submitted by

Letian Leng

Objective

The goal of this assignment is to explore the steady-state thermal conductivity and temperature variations under different material and heat insulation settings. The main object is a casted iron pipe with inside and outside diameters are 70 mm and 90 mm, accordingly.

Model and Geometry

The cast iron pipe (see in Fig.1) has inside and outside diameters are 70 mm and 90 mm, accordingly. Moreover, a heat insulation foam with 5mm thick will be applied to the outside surface of the pipe (see in Fig.2).

| Figure 1. The curved pipe | Figure 2. The pipe with foam insulation (green) | Mesh Settings |
| --- | --- | --- |

Boundary Conditions (Part A)

The thermal conductivity of the pipe is 52 W/m-C, and the pipe is carrying steam at 155 ℃, with an outside temperature of 20 ℃.

We assume that the pipe ends are adiabatic.

The inside surface has a convention coefficient of 20 W/m2 -C, with an outside surface convention coefficient of 3.8 W/m2 -C.

Results (Part A)

A1 – The maximum outside temperature is 128.5 ℃

A2 – The rate of heat loss off the outside surface of the pipe is 35.4 W.

Boundary Conditions (Part B)

The thermal conductivity of the pipe is 52 W/m-C, and the pipe is carrying steam at 155 ℃, with an outside temperature of 20 ℃.

We assume that the pipe ends are adiabatic.

The foam has a thermal conductivity of 0.20 W/m-C, and the inside surface has a convention coefficient of 20 W/m2 -C, with an combined surface convention coefficient of 3.4 W/m2 -C, calculated by:

Results (Part B)

B1 – The maximum outside temperature is 130.8 ℃

B2 – The rate of heat loss off the outside surface of the pipe is 32.3 W.

B3 – The heat loss is smaller than compared to A2, as the heat insulation material prevented some heat loss of that pipe.

Boundary Conditions (Part C)

The thermal conductivity of the pipe is 52 W/m-C, and the pipe is carrying steam at 155 ℃, with an outside temperature of 20 ℃.

We assume that the pipe ends are adiabatic.

The foam has a thermal conductivity of 0.20 W/m-C, and the inside surface has a convention coefficient of 20 W/m2 -C, with an outside surface convention coefficient of 3.8 W/m2 -C, calculated by:

Results (Part C)

C1 – The maximum outside temperature is 118.7 ℃

C2 – The rate of heat loss off the outside surface of the pipe is 35.6 W.

C3 – The heat loss comparison is listed below:

| A2 | B2 | C2 |
| --- | --- | --- |
| 35.4W | 32.3W | 35.6W |

As one can read from the table above that Part B has the lowest heat loss, and Part C has the highest heat loss, one possible reason is due to the increase of surface area (heat insulation material has thickness therefore enlarged the outside surface area), and the influence of surface area overrides the heat insulation effect. Under the situation of outside surface area remain unchanged (Part A and B), the heat insulation does reduce the heat loss.